XGL-FEnet	Software Design Test

# Software Design Test (XGT, FEnet Dedicated Protocol)

First Edition: 2005. 12. 22

Last Edition: 2006. 07. 26 Editor: Daehyun Kwon

Advanced Platform Research Team LSIS R&D Center

1/17

XGL-FEnet	Software Design Test

## Editing History

| Index | Date | Version | Chapter | Revision |
| --- | --- | --- | --- | --- |
| 1 | 2005.12.22 | 0.1 | All | Created |
| 2 | 2005.12.23 | 0.2 | TB, CoS | TB, CoS Tested |
| 3 | 2006.01.07 | 0.3 | 6-2-4-1 | RTC AREA SET Retested because of Offset Field. |
| 4 | 2006.01.10 | 0.4 | 6-3-2 ~ 6-3-4 6-4-2 ~ 6-4-4 | TB, CoS Restart, Stop, Delete is tested. |
| 5 | 2006.06.06 | 0.5 | 6-2-4-2 TB | Device Area Write CMD Corrected. TB Multiple Blocks Test |
| 6 | 2006.06.08 | 0.6 | TB, CoS | TB/CoS Alarm Response Frame : TCP -> UDP |
| 7 | 2006.07.26 | 0.7 | 6-2-4-1 | RTC SET Format is changed to RTC Read Format |

2/17

XGL-FEnet	Software Design Test

# 1. Overview

This document talks about updated functions and revision on Dedicated Protocol of XGT FEnet.

# Frame Structure

The structure of LS Ethernet module’s application frame is as shown below;

| Header |  |
| --- | --- |
| IP Header | TCP Header |

| Application Data Format |  |
| --- | --- |
| Application Header Format | Application Instruction Format |

## Header Structure (Application Header Format)

| Item | Size(byte) | Description |
| --- | --- | --- |
| Company ID | 8 | “LSIS-XGT” (ASCII CODE : 4C 53 49 53 2D 58 47 54) |
| Reserved | 2 | 0x00 : Reserved area |
| PLC Info | 2 | - Client(HMI)  Server(PLC) : Don’t care (0x00) - Server(PLC)  Client(HMI) : - If Reserved 1 is 0x00, Bit00~05 : CPU TYPE 01(XGK-CPUH), 02(XGK-CPUS) Bit06 : 0(Redundant Master / Single), 1(Redundant Slave) Bit07 : 0(CPU operation normal), 1(CPU operation error) Bit08~12 : System status 1(RUN),2(STOP), 4(ERROR), 8(DEBUG) Bit13~15 : Reserved |
| CPU Info | 1 | - Client(HMI)  Server(PLC) : Don’t care (0x00) - Server(PLC)  Client(HMI) : 0xA0 Note1) |
| Source of Frame | 1 | - Client(HMI)  Server(PLC) : 0x33 - Server(PLC)  Client(HMI) : 0x11 |
| Invoke ID | 2 | ID used to discriminate the sequence among frames (This number is sent as attached to Response frame) |
| Length | 2 | Application Instruction’s byte size |
| FEnet Position | 1 | Bit 0~3 : FEnet I/F module’s Slot No. Bit 4~7 : FEnet I/F module’s Base No. |
| Reserved 2 (BCC) | 1 | 0x00 : BCC (Application Header’s Byte Sum) |

3/17

XGL-FEnet	Software Design Test

## Basic Structure of Frame (Application Data Format)

### Command Request frame

(external communication device → FEnet I/F module)

| Header | Command | Data Type | Reserved Area | Structurized data area |
| --- | --- | --- | --- | --- |
|  |  | AREA CODE |  |  |
|  |  | SUB CMD |  |  |

### ACK Response frame

(FEnet module → external communication device, if data is received normally)

| Header | Command | Data Type | Reserved Area | Error status (2 bytes h’0000) | Structurized data area |
| --- | --- | --- | --- | --- | --- |
|  |  | AREA CODE |  |  |  |
|  |  | SUB CMD |  |  |  |

### NAK Response frame

(FEnet module → external communication device, if data is received abnormally)

| Header | Command | Data type | Reserved Area | Error status (2 bytes: h’FFFF) | Error code (2 byte) |
| --- | --- | --- | --- | --- | --- |
|  |  | AREA CODE |  |  |  |
|  |  | SUB CMD |  |  |  |

| Notes |  |
| --- | --- |
| - If hexadecimal data is applied, the Hex. type of data is indicated with ‘h’ or ‘0x’ attached in front of figures inside frame like 01, h’12345, h’34, 0x12 and 0x89AB. - In the Case of NAK Response frame, you can find the meanings of Error codes. Please Refer to Appendix A. |  |

4/17

XGL-FEnet	Software Design Test

# List of Commands

## READ

### 6-1-2) Request Format

6-1-2-1) System Area Read

This function is used to read RTC Data, IO Information, Online History.

| CMD | AREA CODE | Reserved | Size | Offset |
| --- | --- | --- | --- | --- |

| Item | Size(byte) | Description |  |
| --- | --- | --- | --- |
| CMD | 2 | Read Request : 0x1000 |  |
|  |  | Device | 0x0010 |
| Reserved | 2 | 0x0000 : Don’t Care |  |
| Size | 2 | 1 ~ 1400(Bytes) |  |
| Offset | 4 | changeable according to AREA CODE |  |

5/17

XGL-FEnet	Software Design Test

6/17

XGL-FEnet	Software Design Test

6-1-2-2) Device Area Read

This area can be accessible through previous protocol’s separate reading and continuous reading. However, the following functions are added to ensure the overall efficiency.

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Bit Position | Offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Size |  |
| Item |  | Size(byte) | Description |  |  |  |  |
| CMD |  | 2 | Read Request : 0x1000 |  |  |  |  |
| AREA CODE |  | 2 | Device : 0x0010 |  |  |  |  |
| Reserved |  | 2 | 0x0000 : Don’t Care |  |  |  |  |
| # of Block |  | 2 | 1 ~ 64 |  |  |  |  |
| Device Type |  | 1 | P, M, L, K, T, C, D, R, N, ZR(W) ... |  |  |  |  |
| Data Type |  | 1 | X(Bit), B(Byte) |  |  |  |  |
| Bit Position |  | 2 | Bit Position : 0 ~ 7 Size of Block : 1 ~ 1400(Bytes) Size of Block0 + Size of Block1+…+Size of Block64 < 1400 |  |  |  |  |
| Size of Block |  |  |  |  |  |  |  |
| Offset |  | 4 | changeable according to AREA CODE |  |  |  |  |

7/17

XGL-FEnet	Software Design Test

### 6-1-3) Response Format

| CMD | AREA CODE | Reserved | Status | Reserved | Size | Data.. |
| --- | --- | --- | --- | --- | --- | --- |

CF. Previous Dedicated communication service

| CMD | Data Type | Reserved | Status | # of Blocks | # of Data | Data.. |
| --- | --- | --- | --- | --- | --- | --- |

6-1-3-1) System /Device Area Read Response

| Item | Size(byte) | Description |  |
| --- | --- | --- | --- |
| CMD | 2 | Read Response : 0x1001 |  |
|  |  | Device | 0x0010 |
| Reserved | 2 | 0x0000 |  |
| Status | 2 | Normal if it is 0, abnormal if not 0 |  |
| Reserved | 2 | 0x0000 |  |
| Size | 2 | Number of Data, 1 ~ 1400(Bytes) |  |
| Data | Size |  |  |

8/17

XGL-FEnet	Software Design Test

6-1-4-9) Device Area Read

Bit Request Frame

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Bit Position | Offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x1000 | 0x0010 | 0x0000 | 0x0002 | 0x50 | 0x58 | 0x02 | 0x00000000 |
|  |  |  |  | 0x50 | 0x58 | 0x04 | 0x00000000 |

|  | 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 54 ed 87 40 00 80 06 9d 25 96 96 21 6f 96 96 0020 21 5b 10 ea 07 d4 6a cc 79 61 a2 34 80 4d 50 18 0030 3f ec f4 52 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 10 00 18 00 00 b6 00 10 10 00 00 00 0050 02 00 50 58 02 00 00 00 00 00 50 58 04 00 00 00 0060 00 00 |
| --- | --- |

Response Frame

9/17

XGL-FEnet	Software Design Test

Byte Request Frame

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Size | Offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x1000 | 0x0010 | 0x0000 | 0x0006 | 0x44 | 0x42 | 0x04 | 0x00000000 |
|  | D0 | K1 |  | 0x4B | 0x42 | 0x04 | 0x02000000 |
|  | L2 | M3 |  | 0x4C | 0x42 | 0x04 | 0x04000000 |
|  | N4 | P5 |  | 0x4D | 0x42 | 0x04 | 0x06000000 |
|  |  |  |  | 0x4E | 0x42 | 0x04 | 0x08000000 |
|  |  |  |  | 0x50 | 0x42 | 0x04 | 0x0A000000 |

|  | 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 74 ed 89 40 00 80 06 9d 03 96 96 21 6f 96 96 0020 21 5b 10 ef 07 d4 b6 56 66 f3 fa 38 ed fb 50 18 0030 44 70 75 db 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 10 01 38 00 00 d7 00 10 10 00 00 00 0050 06 00 44 42 04 00 00 00 00 00 4b 42 04 00 02 00 0060 00 00 4c 42 04 00 04 00 00 00 4d 42 04 00 06 00 0070 00 00 4e 42 04 00 08 00 00 00 50 42 04 00 0a 00 0080 00 00 |
| --- | --- |

Response Frame

10/17

XGL-FEnet	Software Design Test

Byte Request Frame

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Size | Offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x1000 | 0x0010 | 0x0000 | 0x0006 | 0x52 | 0x42 | 0x04 | 0x0C000000 |
|  | R6 | S7 |  | 0x53 | 0x42 | 0x04 | 0x0E000000 |
|  | T8 | U9 |  | 0x54 | 0x42 | 0x04 | 0x10000000 |
|  | W(ZR)10 | Z11 |  | 0x55 | 0x42 | 0x04 | 0x12000000 |
|  |  |  |  | 0x57 | 0x42 | 0x04 | 0x14000000 |
|  |  |  |  | 0x5A | 0x42 | 0x04 | 0x16000000 |

|  | 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 74 90 7a 40 00 80 06 fa 12 96 96 21 6f 96 96 0020 21 5b 10 ff 07 d4 d4 c1 43 92 d6 a0 d8 bb 50 18 0030 42 40 34 c7 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 10 02 38 00 00 d8 00 10 10 00 00 00 0050 06 00 52 42 04 00 0c 00 00 00 53 42 04 00 0e 00 0060 00 00 54 42 04 00 10 00 00 00 55 42 04 00 12 00 0070 00 00 57 42 04 00 14 00 00 00 5a 42 04 00 16 00 0080 00 00 |
| --- | --- |

Response Frame

11/17

XGL-FEnet	Software Design Test

## WRITE

### 6-2-1) Introduction

·RTC Data SET or Device Data Write Supports;

### 6-2-2) Request Format

6-2-2-2) Device Area Write

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Size | Offset | Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Item | Size(byte) | Description |
| --- | --- | --- |
| CMD | 2 | Write Response : 0x1010 |
| AREA CODE | 2 | Device : 0x0010 |
| Reserved | 2 | 0x0000 |
| # of Block | 2 | 1 ~ 64 |
| Device Type | 1 | P, M, L, K, T, C, D, R, N, ZR(W) |
| Data Type | 1 | X, B |
| Bit Position | 2 | Bit Position : 0 ~ 7 Size of Block : 1 ~ 1400(Bytes) Size of Block0 + Size of Block1+…+Size of Block64 < 1400 |
| Size of Block |  |  |
| Offset | 4 | changeable according to AREA CODE |
| Data | Size of Block | Write Data |
| … |  |  |

12/17

XGL-FEnet	Software Design Test

### 6-2-3) Response Format

| CMD | AREA CODE | Reserved | Status | Error code |
| --- | --- | --- | --- | --- |

6-2-4-2) Device Area Write

Bit Request Frame

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Bit Position | Offset | Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x1010 | 0x0010 | 0x0000 | 0x0002 | 0x50 | 0x58 | 0x01 | 0x00000000 | 00 |
|  |  |  |  | 0x50 | 0x58 | 0x03 | 0x00000000 | 01 |

|  | 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 56 24 6e 40 00 80 06 66 3d 96 96 21 6f 96 96 0020 21 5b 12 a7 07 d4 f8 32 c0 6c d5 56 8f 1b 50 18 0030 42 54 d3 db 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 00 00 1a 00 00 a8 10 10 10 00 00 00 0050 02 00 50 58 01 00 00 00 00 00 01 50 58 03 00 00 0060 00 00 00 01 |
| --- | --- |

Response Frame

| 0000 00 20 ed 53 c1 2f 00 e0 91 02 00 83 08 00 45 00 0010 00 46 35 ab 00 00 80 06 95 10 96 96 21 5b 96 96 0020 21 6f 07 d4 12 a7 d5 56 8e fd f8 32 c0 6c 50 18 0030 3e 80 f1 38 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 01 02 a0 11 00 00 0a 00 00 19 11 10 00 00 10 00 0050 00 00 02 00 |  |
| --- | --- |

13/17

XGL-FEnet	Software Design Test

Byte Request Frame

| CMD | AREA CODE | Reserved | # of Block | Device Type | Data Type | Size | Offset |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x1010 | 0x0010 | 0x0000 | 0x0006 | 0x44 | 0x42 | 0x0002 | 0x00000000 | 0x3412 |
|  | D0 | K1 |  | 0x4B | 0x42 | 0x0002 | 0x02000000 | 0x7856 |
|  | L2 | M3 |  | 0x4C | 0x42 | 0x0002 | 0x04000000 | 0xCDAB |
|  | N4 | P5 |  | 0x4D | 0x42 | 0x0002 | 0x06000000 | 0x10EF |
|  |  |  |  | 0x4E | 0x42 | 0x0002 | 0x08000000 | 0x1211 |
|  |  |  |  | 0x50 | 0x42 | 0x0002 | 0x0A000000 | 0x1413 |

|  | 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 80 b9 50 40 00 80 06 d1 30 96 96 21 6f 96 96 0020 21 5b 0c e3 07 d4 39 bf 90 ee d5 66 2d df 50 18 0030 40 1a 8e 11 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 00 00 44 00 00 d2 10 10 10 00 00 00 0050 06 00 44 42 02 00 00 00 00 00 12 34 4b 42 02 00 0060 02 00 00 00 56 78 4c 42 02 00 04 00 00 00 ab cd 0070 4d 42 02 00 06 00 00 00 ef 10 4e 42 02 00 08 00 0080 00 00 11 12 50 42 02 00 0a 00 00 00 13 14 |
| --- | --- |

Response Frame

| 0000 00 20 ed 53 c1 2f 00 e0 91 02 00 83 08 00 45 00 0010 00 46 ba f7 00 00 80 06 0f c4 96 96 21 5b 96 96 0020 21 6f 07 d4 0c e3 d5 66 2d c1 39 bf 90 ee 50 18 0030 3e 80 42 1b 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 01 02 a0 11 00 00 0a 00 00 19 11 10 00 00 10 00 0050 00 00 06 00 |  |
| --- | --- |

Byte Request Frame

| 0x1010 | 0x0010 | 0x0000 | 0x0006 | 0x52 | 0x42 | 0x0002 | 0x0C000000 | 0x1615 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R6 | S7 |  | 0x53 | 0x42 | 0x0002 | 0x0E000000 | 0x1817 |
|  | T8 | U9 |  | 0x54 | 0x42 | 0x0002 | 0x10000000 | 0x1A19 |
|  | W(ZR)10 | Z11 |  | 0x55 | 0x42 | 0x0002 | 0x12000000 | 0x1C1B |
|  |  |  |  | 0x57 | 0x42 | 0x0002 | 0x14000000 | 0x1E1D |
|  |  |  |  | 0x5A | 0x42 | 0x0002 | 0x16000000 | 0x201F |

|  | 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 74 90 7a 40 00 80 06 fa 12 96 96 21 6f 96 96 0020 21 5b 10 ff 07 d4 d4 c1 43 92 d6 a0 d8 bb 50 18 0030 42 40 34 c7 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 10 02 38 00 00 d8 00 10 10 00 00 00 0050 06 00 52 42 04 00 0c 00 00 00 53 42 04 00 0e 00 0060 00 00 54 42 04 00 10 00 00 00 55 42 04 00 12 00 |
| --- | --- |

14/17

XGL-FEnet	Software Design Test

| 0000 00 e0 91 02 00 83 00 20 ed 53 c1 2f 08 00 45 00 0010 00 80 76 a0 40 00 80 06 13 e1 96 96 21 6f 96 96 0020 21 5b 0c e3 07 d4 39 ff 0a ec d5 7b f4 99 50 18 0030 41 0a 56 22 00 00 4c 53 49 53 2d 58 47 54 00 00 0040 00 00 00 33 00 00 44 00 00 d2 10 10 10 00 00 00 0050 06 00 52 42 02 00 0c 00 00 00 15 16 53 42 02 00 0060 0e 00 00 00 17 18 54 42 02 00 10 00 00 00 19 1a 0070 55 42 02 00 12 00 00 00 1b 1c 57 42 02 00 14 00 0080 00 00 1d 1e 5a 42 02 00 16 00 00 00 1f 20 | 0070 00 00 57 42 04 00 14 00 00 00 5a 42 04 00 16 00 0080 00 00 |
| --- | --- |

Response Frame

15/17

XGL-FEnet	Software Design Test

# Appendix

### A. Details of error code hex 2 Bytes

| Error Code | Error Type | Error details and causes | Remarks |
| --- | --- | --- | --- |
| 0x00 | Frame Error | BCC Error |  |
| 0x01 |  |  |  |
| 0x02 | Command Error | Unknown Command |  |
| 0x03 | Sub Command Error | Unknown Sub Command |  |
| 0x04 | Area Address Error | Request exceeds the area each device supports. |  |
| 0x05 | Data Value Error | Request abnormal Data (RTC Data, Bit Pos..) |  |
| 0x10 | Data size error | Request exceeds the max range of 1400 Bytes to read or write at a time. |  |
| 0x11 | Data type error | Other data type than X,B received. |  |
| 0x12 | Device type Error | Other device type than P, M, L, K, T, C, U, D, N, R. W. received. |  |
| 0x13 | Number of blocks exceeded | Number of blocks exceeds 64 at Read/Write Request Number of blocks exceeds 40 at TB/CoS Request |  |

16/17

XGL-FEnet	Software Design Test

17/17